Project Proposal

For Consideration Under

TIH IoT Technology Development Program 2026

TankGuard: self-powered monitoring, pump control and sludge cleaning for water tanks

Submitted by

Dr. Vishnu Vasudev

Sree Buddha College of Engineering

Industry partner: Nuke Labs

Theme: TIH IoT Technology Development Program 2026

Date of submission: 31 October 2026

# Contents

1  Cover Sheet	3

2  Executive Summary	3

3  Background	3

3.1  History	3

3.2  Statement of the problem	4

3.3  Objectives	4

3.4  Deliverables	4

3.5  Project plan with major milestones	5

3.6  Resources and budget	5

3.7  Category of new technology/product	6

3.8  Commercialisation plan	6

4  Appendix	6

4.1  Supporting documentation	6

# 1  Cover Sheet

| Field | Detail |
| --- | --- |
| Project title | TankGuard: self-powered monitoring, pump control and sludge cleaning for water tanks |
| Principal investigator | Dr. Vishnu Vasudev |
| Designation | Principal Investigator |
| Affiliation | Sree Buddha College of Engineering, Department of Electronics and Communication Engineering |
| Co-investigator | None |
| Collaborator | Nuke Labs (build, field access, commercialisation) |
| Theme | TIH IoT Technology Development Program 2026 |
| Date of submission | 31 October 2026 |

# 2  Executive Summary

TankGuard is a self-powered monitoring and cleaning system for water storage tanks in homes and institutions. It measures water quality in the tank, reports one score, controls the household pump, and cleans the sludge that settles on the floor. All of it runs on the local network, with no cloud service, subscription or account.

The system has three units. M1 sits on the tank lid and reads turbidity, TDS, pH, temperature and water level through submerged probes. It computes a weighted score between 0 and 100 and serves the dashboard from its own flash memory. H1 sits next to the pump switch, shows the score on a 1.8 inch colour display, and drives the pump through an isolated relay and contactor. C1 is the cleaning arm: two rigid links of 20 cm each, a ring brush at the tip, and a submersible pump that draws the loosened sludge through the centre of the head.

Tanks degrade without warning. Disinfectant residual is gone within 24 to 48 hours of filling. Particles settle into a sludge layer at the floor, biofilm forms on the walls, and algae grow faster in warm water. The water is unsafe before it smells or looks wrong. The usual remedy is a full drain and a manual scrub, which discards the whole stored volume.

TankGuard measures, decides and cleans at the edge. One cleaning cycle covers 90 percent of the floor. A 2 cm ring at the wall stays out of reach, and the design record states it. The dashboard reports the data source and the coverage limit, so the owner knows what was measured and what was cleaned.

The monitoring prototype is built. The M1 firmware, the dashboard and the path planner run, and INR 23,587 of parts have already been invested. The grant funds the step from that prototype to a certified product: the arm hardware, twenty-five pilot units, certification, a patent and twelve field installations.

The project delivers a production revision of M1 and H1, twenty-five pilot units of the C1 cleaning arm, twelve field installations, certification, a filed patent, and three paying pilot customers. The target is TRL 8, with the unit cost and the service model proven before the first production run.

# 3  Background

## 3.1  History

TankGuard started as a water-quality monitor. Five probes fed an edge-computed score, and the ESP8266 served its own dashboard over Wi-Fi, with no cloud service.

Field trials exposed the real failure mode. Measurement alone does not clean a tank: the sludge layer keeps feeding turbidity and biofilm, and the household still drains and scrubs by hand. The product moved from monitoring to care, with a home unit for the pump and an arm for the sludge.

The current design keeps the five probes, the score weights and the local-only architecture. It replaces the abandoned drain-and-flush mechanism with the cleaning arm described here.

## 3.2  Statement of the problem

Storage tanks degrade silently. Residual chlorine disappears within 24 to 48 hours, suspended solids settle into a sludge layer, biofilm forms on internal surfaces, and algae grow in warm water. By the time it is visible, the water is already unsafe.

Current options either monitor without acting or clean by draining the whole tank. Monitoring tells the owner there is a problem and leaves the work to them. A full drain wastes the stored water and needs manual labour.

The gap is largest where it matters most: schools, hospitals and apartment blocks store large volumes, clean to a fixed calendar, and cannot tell whether the last clean helped.

## 3.3  Objectives

Objective 1: measure and score water quality at the edge. Five probes (turbidity, TDS, pH, temperature, level) are digitised through an ADS1115 and fused into a weighted score (40/25/25/10). Acceptance: the score repeats within 2 points over ten consecutive samples in still water.

Objective 2: make the result readable without a phone. H1 shows the score, level, the four readings, pump state and link health on a 1.8 inch display. It controls the pump in automatic mode (on below the low-level setpoint, off above the high) with a manual override that releases safely.

Objective 3: clean the tank floor without draining it. C1 sweeps three concentric rings at 8.0, 23.7 and 39.4 cm with a 16 cm ring brush and a central suction intake, covering 90 percent of the floor per cycle, and parks in a fixed rest pose at both ends of the cycle.

Objective 4: prove it in the field and take it to market. The monitor enters at TRL 6 and the arm at TRL 4; twelve months later the system is at TRL 8, with twenty-five pilot units, twelve installations, certification, a filed patent and three paying customers.

## 3.4  Deliverables

D1. M1 monitor, production revision: the ESP8266 board with five calibrated probes, an edge score, the dashboard in flash, and an enclosure that can be moulded at volume.

D2. H1 home unit, production revision: the display board, isolated relay and contactor interface, and a manual override that passes electrical safety review.

D3. C1 cleaning arm, pilot build: twenty-five units with the two-link arm, ring brush, suction head and the planner-generated path, at a confirmed unit cost.

D4. Commercial pack: twelve installations, certification and patent filed, three paying pilot customers, a channel agreement, and the TRL 8 evidence set.

## 3.5  Project plan with major milestones

Twelve months from T0, with a deliverable at every gate.

| # | Milestone | Period | Delivered at the gate |
| --- | --- | --- | --- |
| 1 | Design freeze and tooling | T0 + 3 months | Arm and enclosure frozen. Moulds ordered. Five pilot sets assembled. Certification scope agreed with the test laboratory. |
| 2 | Pilot build and lab qualification | T0 + 6 months | Twenty-five pilot sets built and calibrated against reference instruments. IP68 and EMC pre-compliance passed. Unit cost confirmed at INR 15,600 or below. |
| 3 | Field deployment | T0 + 9 months | Twelve installations in three cities with sixty days of continuous operation. Installation manual and service workflow complete. |
| 4 | Commercial readiness | T0 + 12 months | Certification filed, patent filed, three paying pilot customers, channel agreement signed, TRL 8 evidence pack. |

## 3.6  Resources and budget

| Item | Amount (INR) |
| --- | --- |
| Manpower | 12,00,000 |
| Equipment (exclusive of GST) | 8,40,000 |
| Consumables | 4,40,000 |
| Travel | 2,00,000 |
| Miscellaneous | 4,40,000 |
| Contingency (max 5%) | 1,56,000 |
| Sub total | 32,76,000 |
| Overhead (max 20%) | 6,55,200 |
| Faculty fee (max 20%) | 3,00,000 |
| Total (7+8+9) | 42,31,200 |
| GST (18%) | 0 |
| Grand total | 42,31,200 |

Manpower covers two full-time engineers at Nuke Labs for twelve months, one mechanical and one firmware, at INR 50,000 per month each. Equipment covers enclosure tooling (INR 4.0 lakh), a tank test rig (INR 2.0 lakh) and reference instruments for validation (INR 2.4 lakh). Consumables cover twenty-five pilot sets at INR 15,600 each, plus PCB fabrication and spares.

Miscellaneous covers certification (INR 2.0 lakh), patent drafting and filing (INR 1.4 lakh), and market work with channel partners (INR 1.0 lakh). Travel covers the field installations and review meetings (INR 2.0 lakh). Contingency is 5 percent of direct cost and overhead 20 percent of the sub total; the faculty fee is INR 3.0 lakh, below the 20 percent cap.

Prototype parts to date total INR 23,587, already invested and not part of this request.

Tranche disbursement schedule

| Timeline | Amount (INR) | Milestone covered |
| --- | --- | --- |
| Project start (T0) | 15,69,160 | Design freeze, tooling and the first pilot sets |
| T0 + 6 months | 16,39,560 | Pilot build, certification testing and field deployment |
| T0 + 12 months | 10,22,480 | Validation close-out, IP filing and commercial launch |
| Total | 42,31,200 |  |

## 3.7  Category of new technology/product

New-to-the-world product. The combination of edge-computed water scoring, a dashboard hosted on the device, safe pump control and a selective cleaning arm on one local network is not offered by monitoring products or by full-drain cleaning services. The closest alternatives cover one part of the problem.

## 3.8  Commercialisation plan

The buyer is the contractor or facility manager who already pays for cleaning. A household buys a clean, not a device, so the arm is sold to the contractor who visits the tank, and the monitor is sold to the institution that wants a reading between cleans.

Price targets: INR 39,999 for the complete system (M1, H1 and C1) against INR 15,600 of parts, a gross margin near 61 percent. The monitor and home unit alone sell at INR 16,999 for sites that do not need the arm.

Market base: 79 million urban households in the 2011 Census, of which about 60 percent are assumed to store water in a tank (47 million), plus 1.5 million schools and 70,000 hospitals. A tank clean sells for INR 1,500 to 4,000 in urban India, once or twice a year.

Route to revenue: three paying pilot sites in the grant year, converted to reference accounts; two channel partners in the following year, one in cleaning services and one in plumbing distribution; a target of 250 units in the first full year of sale.

The grant pays for the tooling, certification and pilot units that a first customer will not fund. Nuke Labs carries the channel, the service network and the field access.

# 4  Appendix

## 4.1  Supporting documentation

TankGuard Technical Specification, document TG-SPEC-2026-001, October 2026, 7 pages.

Firmware for all three units, compiling for ESP8266 (M1 and H1) and ESP32 (C1), with the React dashboard served from M1 flash.

Design record (docs/CLEANER.md): kinematics, coverage analysis and control limits, including the 2 cm wall ring the arm cannot reach.

Coverage planner (tools/swept_area.html) and the generated path used by the controller (firmware/c1_cleaner/path.h).

Patentability disclosure and the minimum bill of materials.

Prototype spend to date: INR 23,587 in parts, with the monitor running at TRL 6.